Rolling Scaffold Safety

The following rules apply:

Do not ride rolling scaffolds.

- Remove all material and equipment from the platform before moving the scaffold.

- Caster brakes must be applied at all times when scaffolds are not being moved.

- Do not attempt to move a rolling scaffold without sufficient help. Watch out for holes in the floor and overhead obstructions.

- Do not extend adjusting screws on rolling scaffolds that are more than 12 inches.

- Use horizontal-diagonal bracing near the bottom, top, and at intermediate levels of 30 inches.

- Do not use brackets on rolling scaffolds without consideration of the overturning effect.

- The working platform height of a rolling scaffold must not exceed four times the smallest base dimension unless guyed or otherwise stabilized.

For putlogs and trusses, the following additional rules apply:

- Do not cantilever or extend putlogs/trusses as side brackets without thorough consideration for loads to be applied.

- Putlogs/trusses should extend at least 6 inches beyond the point of support.

- Place proper bracing between putlogs/trusses when the span of putlogs/trusses is more than 12 inches.

SAFETY TALK SIGN IN SHEET

This form documents that the training specified above was presented to the listed participants. By signing below, each participant acknowledges receiving this training.

Organization:

Trainer:					 Trainer’s Signature:

Class Participants:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date:

Name:					 Signature:				 Date: